NATIONAL TECHNICAL UNIVERSITY OF UKRAINE

“KYIV POLYTECHNIC INSTITUTE”

FACULTY OF HEAT POWER ENGINEERING

DEPARTMENT OF AUTOMATION OF POWER PROCESSES AND

SYSTEMS ENGINEERING

COURSEWORK

course: “Visualization of graphical and geometrical information”

theme: “Tangents and a normal to a point lying on the shoe surface”

Variant 12

Presented to:

Anatolii Demchyshyn

Prepared by:

Dmytro Kruhlyi

5th year student

Group TR-02mp

Kyiv – 2020

Task:

- Calculate tangents in a point on a surface using either numerical or numeric partial derivatives.

- Calculate a normal vector taking cross product of the two derivatives.

- It has to be possible to move the point along the surface (u,v) space using a keyboard. E.g. keys A and D move the point along u parameter and keys W and S move the point along v parameter.

Theory:

A Monge surface with a cylindrical directrix surface and a sinusoid curve as meridian may be defined as a surface formed by a moving sinusoid, the plane of which rolls without slipping above a right circular cylinder. Assuming a sinusoid as a generatrix curve or a meridian and an evolvent of the circle as a directrix curve or a parallel, it is possible to give another determination for a studied Monge surface: Monge surface with a cylinder directrix surface and a sinusoidal generatrix is formed by a plane sinusoid moving along a directrix evolvent of the circle, so that a plane with the generatrix sinusoid lies all the time at the normal plane of directrix evolvent of the circle and is deadly linked with it.

Parametrical equations (Fig. 1):

Fig. 1

The surface is given in lines of the principal curvatures t, α. One family of the plane lines of principal curvatures denoted by t coincides with the generatrix sinusoids, but the second family of the plane lines of principal curvatures α is the family of the evolvents of the circle with a radius r.

To get tangents vectors we can use numeric partial derivative:

tu=(dxdu; dydu; dzdu)

tv=(dxdv; dydv; dzdv)

To get a normal vector we can use inner product [tu* tv] :

a* b= a1* b1+ a2* b2+ a3* b2

Implementation:

For implementation this surface and tangents with normal I used JavaScript language and three.js library. Three.js library allow us to get 3D content on a webpage and it’s very easy to use it.

The process of rendering consists of 7 parts:

- Create a scene and a camera using THREE.PerspectiveCamera and  THREE.Scene classes.

- Create custom mongeSurfaceDiretrixSinusoidaMeridian function and new geometry using THREE.ParametricGeometry. Add it to new THREE.Mesh and add this mesh to scene.

- Create functions which use numeric partial derivatives to calculate tangents vectors.

- Create a function to calculate a normal vector.

- Create three objects of THREE.LineBasicMaterial to visualize tangents vectors and a normal.

- Add Event Listener to the document to move dots on a surface using keys “A”, “W”, “S” and “D”. Кeys “A” and “D” move the point along u parameter and keys “W” and “S” move the point along v parameter. Point movement is limited to the limits of the function.

- Render scene using THREE.WebGLRenderer.

Screenshots:

The screenshots show an example of point movement. The tangent U vector is green, the tangent V vector is red and the normal vector is white.

Source code:

The source code of my program is shown below.

const canvas = document.getElementById("canvas");

const scene = new THREE.Scene();

const camera = new THREE.PerspectiveCamera(75, window.innerWidth / window.innerHeight, 0.1, 1000);

camera.position.set(0, 0, -10);

camera.lookAt(0, 0, 0);

const renderer = new THREE.WebGLRenderer({ canvas: canvas });

renderer.setSize(window.innerWidth, window.innerHeight);

renderer.setClearColor(0x000000);

const delta = 0.001;

let mainNormal, firstTangent, secondTangent;

const geometry = new THREE.ParametricGeometry(mongeSurfaceDiretrixSinusoidaMeridian, 100, 100);

const material = new THREE.MeshNormalMaterial();

const kleinBottle = new THREE.Mesh(geometry, material);

scene.add(kleinBottle);

const gui = new dat.GUI();

const box = gui.addFolder('Switcher');

box.add(kleinBottle.material, 'wireframe').listen();

box.open();

var movingU = 0.5;

var movingV = 0.5;

const controls = new THREE.OrbitControls(camera, renderer.domElement);

const light = new THREE.AmbientLight(0xffffff);

scene.add(light);

drawingTangentsAndNormal(movingU, movingV);

renderer.setAnimationLoop(() => { renderer.render(scene, camera); })

document.addEventListener('keydown',(event) => {

event.preventDefault();

switch (event.code) {

case "KeyA":

movingU -= 0.01;

drawingTangentsAndNormal(movingU, movingV);

break;

case "KeyD":

movingU += 0.01;

drawingTangentsAndNormal(movingU, movingV);

break;

case "KeyW":

movingV += 0.01;

drawingTangentsAndNormal(movingU, movingV);

break;

case "KeyS":

movingV -= 0.01;

drawingTangentsAndNormal(movingU, movingV);

break;

}

});

function drawingTangentsAndNormal(alpha, t) {

alpha *= 10;

t *= 10;

let tv = new THREE.Vector3(dxdv(alpha,t), dydv(alpha,t), dzdv(alpha,t));

let tu = new THREE.Vector3(dxdu(alpha,t), dydu(alpha,t), dzdu(alpha,t));

let normal = new THREE.Vector3(0,0,0).copy(tv).crossVectors(tv, tu);

let vect = new THREE.Vector3(calcX(alpha, t), calcY(alpha, t), calcZ(alpha, t));

let geomTv = new THREE.Geometry();

geomTv.vertices.push(tv, vect);

let geomTu = new THREE.Geometry();

geomTu.vertices.push(tu, vect);

let geomNormal = new THREE.Geometry();

geomNormal.vertices.push(normal, vect);

if(mainNormal == null) {

firstTangent = new THREE.Line(geomTv, new THREE.LineBasicMaterial({ color: 0xFF0000 }));

secondTangent = new THREE.Line(geomTu, new THREE.LineBasicMaterial({ color: 0x00FF00 }));

mainNormal = new THREE.Line(geomNormal, new THREE.LineBasicMaterial({ color: 0xFFFFFF }));

}

firstTangent.geometry = geomTv;

secondTangent.geometry = geomTu;

mainNormal.geometry = geomNormal;

scene.add(firstTangent);

scene.add(secondTangent);

scene.add(mainNormal);

}

function dxdv(u, v){

return (calcX(u, v + delta * v) - calcX(u, v)) / delta;

}

function dydv(u, v){

return (calcY(u, v + delta * v) - calcY(u, v)) / delta;

}

function dzdv(u, v){

return (calcZ(u, v + delta * v) - calcZ(u, v)) / delta;

}

function dxdu(u, v){

return (calcX(u + delta * u, v) - calcX(u, v)) / delta;

}

function dydu(u, v){

return (calcY(u + delta * u, v) - calcY(u, v)) / delta;

}

function dzdu(u, v){

return (calcZ(u + delta * u, v) - calcZ(u, v)) / delta;

}

function calcX(alpha, t){

return  1 * Math.cos(alpha) -

(t * Math.cos(Math.PI * 0.2) - 2 * Math.sin(1 * t) * Math.sin(Math.PI * 0.2) + t * Math.cos(Math.PI * 0.2) -

2 *  Math.sin(1 * t) * Math.sin(Math.PI * 0.2)) * Math.sin(alpha);

}

function calcY(alpha, t){

return 1 * Math.sin(alpha) +

(t * Math.cos(Math.PI * 0.2) - 2 * Math.sin(1 * t) * Math.sin(Math.PI * 0.2) + t * Math.cos(Math.PI * 0.2) -

2 *  Math.sin(1 * t) * Math.sin(Math.PI * 0.2)) * Math.cos(alpha);

}

function calcZ(alpha, t){

return t * Math.sin(Math.PI * 0.2) + 2 * Math.sin(1 * t) * Math.cos(Math.PI * 0.2);

}

function mongeSurfaceDiretrixSinusoidaMeridian(alpha, t, target){

alpha *= 10;

t *= 10;

let x = calcX(alpha, t);

let y = calcY(alpha, t);

let z = calcZ(alpha, t);

return target.set(x, y, z);

}

Conclusions:

After implementation this course work I improved my skills with Three.js library and also learned to build tangents and normal to a given surface.